Automatic Lane Change Decision Making 
using Imitation Learning

Li-Ta Lai, Chuan-Yu Yan, Bo-Chiuan Chen

National Taipei University of Technology, Taipei, Taiwan

E-mail: bochen@mail.ntut.edu.tw

In this paper, the decision-making of an automatic lane change system using imitation learning is proposed to mimic a human driver’s lane change decision on the road. SUMO is used to establish scenarios with different traffic flows for collecting the driving data of the built-in driver model. Whenever lane change happens, the dynamics of the host vehicle and the relative kinematics of surrounding vehicles are recorded and converted into time headway, time-to-collision, and relative velocity, which are treated as three input layers for a 2D convolutional neural network. When the network is uncertain about the decision, a rule-based risk assessment is used to assist the lane change decision-making. Simulation results show that the proposed algorithm can determine the correct timing for lane change. No collision is detected for test maneuvers.

Topics / Autonomous Driving Systems, Advanced Driver Assistance Systems

1. INTRODUCTION

With the growth of the economy, the amount of vehicles is increasing. Traffic accidents have become the eighth leading cause of death in the world. According to the statistics and analysis of NHTSA's accident data [1], lane-changing accidents account for 10% of the total accidents, and 75% of drivers are not aware of the danger. It is also pointed out that 57% of lane-changing and overtaking accidents are caused by driving factors [2].

Decision-making research about lane change can be divided into two approaches, rule-based and learning-based. For the rule-based approach, Brännström et al. [3] define the shape of the vehicle as a rectangle and estimate how the driver should maneuver to avoid danger in the event of a rear-end collision. Time-to-collision (TTC) is used as the trigger timing of automatic emergency braking. Labayrade et al. [4] proposed to use the TTC to evaluate the current driving behavior and control the vehicle with the minimum collision risk. When the vehicle is changing lanes, Lee et al. [5] proposed safe boundaries for the front and rear of the vehicle. A Kalman filter is used to estimate the relative distance and relative speed of the vehicle in the adjacent lane. The overlap between the safe boundaries and the predicted relative distance is used to determine the risk of the lane-changing behavior. Jula et al. [6] proposed a minimum longitudinal distance that the vehicle should have initially to ensure that no collision occurs during the lane change.

For the learning-based approach, Codevilla et al. [7] proposed a conditional end-to-end imitation learning algorithm, which uses external driving control commands and trains the corresponding decision-making control network for different driving conditions. Liu et al. [8] proposed a decision-making model for lane change based on the benefit, safety, and tolerance. A Support Vector Machine (SVM) is used to solve the related multi-parameter and nonlinear problem. Hole et al. [9] proposed a decision-making model based on the Convolutional Neural Network (CNN) architecture for lane change. The relative distance, speed, and lane position of surrounding vehicles are used as the inputs of the decision-making network. Yang et al. [10] proposed a vision-based Long Short-Term Memory (LSTM) and Recurrent Neural Network (RNN) intent inference system to imitate driving intent in advance through vehicle dynamics and visual images from the driver’s perspective. Zhang et al. [11] proposed a deep imitation learning framework (CNN-RNN-Attention) based on the sensor data to predict the driving intention to avoid the driver's privacy problem.

In order to prevent possible aggressive decisions from a 2D CNN, a rule-based risk assessment is used to assist the lane change decision in this paper as shown in Fig. 1. The remainder of the paper is organized as follows.  In Section 2, the 2D CNN is introduced. The proposed decision-making is presented in Section 3. Simulation results are illustrated and analyzed in Section 4. Conclusions are presented in Section 5.

2. 2D CNN

2.1 Data Collection

SUMO is used to simulate the virtual traffic flow as shown in Fig. 2. One thousand driving simulations are executed. Initial speed and position of each vehicle are randomly specified for each simulation. When the SUMO's driver has the intent to change lane, the turn signal is triggered as shown in Fig. 3. If non-zero lateral displacement is observed, the event is labeled as lane change. However, if there is no lateral displacement, the event is labeled as lane keeping.

Fig. 1 Proposed lane change decision making

In order to increase the data diversity, total events are augmented by flipping the original events along the longitudinal axis of the vehicle. The collected lane change (LC) events are shown in Table 1 The numbers of the three types of events are balanced for training the decision network.

Fig. 2 Virtual traffic flow in SUMO

| a. Lane change event | b. Lane keeping event |
| --- | --- |

Fig. 3  Data labeling

Table 1  Lane change events

| Event | Amount | After Augmentation |
| --- | --- | --- |
| LC to Left (LCL) | 1250 | 1734 |
| Lane Keeping (LK) | 454 | 908 |
| LC to Right (LCR) | 484 | 1734 |

2.2  Risk Assessment Regions

In this paper, the reference [12]  is applied to divide the driving lane and adjacent lanes into 8 risk assessment regions around the host vehicle as shown in Fig. 4. Wr is the lane width, G2 is the vehicle length, G1 is the risk assessment distance in front of the vehicle and G3 is the risk assessment distance behind the vehicle. G1 and G3 are shown in equations (1) and (2), respectively.

Fig. 4 Risk assessment region

G1=THset×vx,h            		  (1)

G3=THset×vmax             	  (2)

where THset is the set time headway, vx,h is vehicle speed, and vmax is the maximum vehicle speed set in the current lane. Whenever the driving speed increases, the length of the front risk assessment area is increased. The time headway (TH) and TTC of surrounding vehicles in the target and current lanes are calculated according to the estimated relative kinematics.

2.3 Network Structure

In order to imitate the driver's lane changing behavior, a 2D CNN, which consists of a CNN layer and two fully connected layers, is used to design the decision network as shown on the left in Fig. 1. TH, TTC, and relative velocity (RV) of surround vehicles in the 8 regions around the host vehicle are treated as three input layers for CNN. RV is defined as the velocity of the detected vehicle minus the velocity of the host vehicle. The decision-making output is lane change to left (LCL), lane keeping (LK), or lane change to right (LCR).

The confusion matrix of the 2D CNN approach is shown in Table 2 The accuracy is 99.14%. When the actual decision from the SUMO driver model is lane keeping, 9 (7+2) predicted decisions from the decision network are lane changes. It might be too aggressive or dangerous to execute these inconsistent predicted decisions. On the other hand, it is safe to be conservative when 17 (13+4) predicted lane keeping decisions are inconsistent with actual lane change decisions.

Table 2 Confusion matrix of 2D-CNN

| Accuracy 99.14% |  | Predicted |  |  |
| --- | --- | --- | --- | --- |
|  |  | LCL | LK | LCR |
| Actual | LCL | 1267 | 13 | 0 |
|  | LK | 7 | 445 | 2 |
|  | LCR | 0 | 4 | 1276 |

Since the aggressive lane change decision might be possible to cause a collision, the associated confidence level of the prediction results is further analyzed. Fig. 5 shows the confidence levels for both wrong and correct predictions. It can be observed that the confidence levels are low for wrong predictions, and the confidence levels are high for correct predictions.

Fig. 5 Confidence level of LC decisions from 2D CNN

3. DECISION MAKING

In order to prevent possible aggressive predicted decisions, the rule-based risk assessment as shown in Fig. 6 is used to assist the lane change decision. If the confidence levels of LCL and LCR decisions are lower than the confidence threshold CONFt which is set at 85% in this paper, it is determined to be uncertain to activate lane change. TH and TTC are then used to determine if it is safe for lane change.

Fig. 6 Rule-based risk assessment

The confusion matrix with a 2D CNN and a rule-based risk assessment is shown in Table 3. The accuracy can be increased to 99.44%. Nine aggressive decisions listed in Table 2 are now relocated as LK in Table 3. Thus, no aggressive lane change decisions are observed while keeping the same 17 conservative decisions.

Table 3 Confusion Matrix of Proposed Approach

| Accuracy 99.44% |  | Predicted |  |  |
| --- | --- | --- | --- | --- |
|  |  | LCL | LK | LCR |
| Actual | LCL | 1267 | 13 | 0 |
|  | LK | 0 | 445+9 | 0 |
|  | LCR | 0 | 4 | 1276 |

4. SIMULATION RESULT

CarSim is used to evaluate the proposed algorithm in Matlab/Simulink. The structure of the automatic lane change system used in this paper is shown in Fig. 7. Relative positions and velocities of surrounding vehicles are obtained from the SUMO interface. Collision risks with surrounding vehicles are assessed via estimating the relative kinematics.  TH, TTC, and RV are used as the feature inputs of the lane change decision making. If it is determined to be safe, a lane change trajectory is designed using trapezoidal acceleration profile with the acceleration limits for ride comfort [13, 14]. Adaptive Cruise Control and Lateral Displacement Control are employed to follow the lane change trajectory. Two lane change events with high confidence level and low confidence level are discussed in this section.

Fig. 7 Automatic Lane Change System

4.1 High Confidence Level

The initial conditions of surrounding vehicles for the event with high confidence level are shown in Fig. 8. Lane change responses of three vehicles are shown in Fig. 9. TH, TTC, and RV are expressed from the perspective of the host vehicle. The driving range of the host vehicle during lane change is from 0 to 99 m. TH and TTC of the MF vehicle indicate that there is enough space for lane change before crossing the lane. When the host vehicle enters the target lane, TH of the RR vehicle is still increasing due to that the velocity of host vehicle is higher than that of the RR vehicle. Finally, the host vehicle finishes lane change at 99m safely without any collision.

Fig. 8 Initial conditions for high confidence level

Fig. 9 Responses for high confidence level

4.2 Low confidence level

The initial conditions of surrounding vehicles for the event with low confidence level is shown in Fig. 10. Lane change responses of three vehicles are shown in Fig. 11. The driving range of the host vehicle during lane change is from 0 to 118 m. TH and TTC of the MF vehicle indicate that there is enough space for lane change before crossing the lane. After the host vehicle enters the target lane, the SUMO driver model of the RR vehicle detects the cut-in host vehicle with slower velocity and starts to decelerate’. However, the TH of the RR vehicle is barely maintained around 0.4 sec. Although no collision occurs during the lane change, it might impose the visual threat to the passengers on the RR vehicle.

Fig. 10 Initial conditions for low confidence level

Fig. 11 Responses for low confidence level

If the driver of the RR vehicle is distracted and does not decelerate to avoid the collision with the cut-in of the host vehicle as shown in Fig. 12, TH and TTC of the RR vehicle are decreased gradually. Finally, the collision of the host vehicle and the RR vehicle occurs at 122m.

Fig. 12 Responses for low confidence level with no deceleration

However, the proposed approach can employ the rule-based strategy to further evaluate the lane change decision with low confidence. As can be seen from Fig.12, both TH and TTC of the RR vehicle are lower than the thresholds THth and TTCth which are set to be 0.7 sec and 2.5 sec, respectively. Thus, the proposed approach can effectively detect the collision risk, and determine to keep in the lane as shown in Fig. 13 to avoid the possible collision.

Fig. 13 Trajectory response of the proposed approach for low confidence level

5. CONCLUSION

Simulation results show that the proposed algorithm, which consists of a 2D CNN and a rule-based risk assessment, can effectively prevent aggressive decisions of lane change and generate correct and safe lane change decisions close to those of the virtual driver in SUMO. Thus, the proposed algorithm can be used to imitate the driver’s decision-making to interact with surrounding vehicles on road.

REFERENCES

- S.E. Lee, E.C.B. Olsen, and W.W. Walter, “A Comprehensive Examination of Naturalistic Lane-Changes,” National Highway Traffic safety Administration, March 2004.

- The Royal Society for the Prevention of Accidents, “Road Safety Factsheet,” July 2017.

- Z. Chen and X. Huang, “End-to-end learning for lane keeping of self-driving cars,” 2017 IEEE Intelligent Vehicles Symposium (IV), Los Angeles, CA, pp. 1856-1860, 2017.

- Y. Liu, X. Wang, L. Li, S. Cheng, and Z. Chen, “A Novel Lane Change Decision-Making Model of Autonomous Vehicle Based on Support Vector Machine,” in IEEE Access, vol. 7, pp. 26543-26550, 2019.

- C. Vallon, Z. Ercan, A. Carvalho, and F. Borrelli, “A machine learning approach for personalized autonomous lane change initiation and control,” 2017 IEEE Intelligent Vehicles Symposium (IV), pp. 1590-1595, 2017.

- C. Hoel, K. Wolff, and L. Laine, “Automated Speed and Lane Change Decision Making using Deep Reinforcement Learning,” 2018 21st International Conference on Intelligent Transportation Systems (ITSC), pp. 2148-2155, 2018.

- F. Codevilla, M. Miiller, A. López, V. Koltun and A. Dosovitskiy, “End-to-End Driving Via Conditional Imitation Learning,” 2018 IEEE International Conference on Robotics and Automation (ICRA), pp. 4693-4700, 2018.

- Y. Liu, X. Wang, L. Li, S. Cheng, and Z. Chen, “A Novel Lane Change Decision-Making Model of Autonomous Vehicle Based on Support Vector Machine,” in IEEE Access, vol. 7, pp. 26543-26550, 2019.

- C. Hoel, K. Wolff, and L. Laine, “Automated Speed and Lane Change Decision Making using Deep Reinforcement Learning,” 2018 21st International Conference on Intelligent Transportation Systems (ITSC), pp. 2148-2155, 2018.

- X. Yang, L. Chen, W. Huaji, C. Dongpu, and V. Efstathios, “An Ensemble Deep Learning Approach for Driver Lane Change Intention Inference,” Transportation Research Part C: Emerging Technologies, June 2020.

- J. Zhang, Z.C. Wu, F. Li, C. Xie, T. Ren, J. Chen and L. Liu, “A Deep Learning Framework for Driving Behavior Identification on In-Vehicle CAN-BUS Sensor Data,” Sensors, March 2019.

- H. Tehrani, Q. H. Do, M. Egawa, K. Muto, K. Yoneda, and S. Mita, “General behavior and motion model for automated lane change,” 2015 IEEE Intelligent Vehicles Symposium (IV), Seoul, pp. 1154-1159, 2015.

- International standard orginization, “Intelligent transport systems — Adaptive Cruise Control systems — Performance requirements and test procedures,” 2002.

- International standard orginization, “Intelligent transport systems — Lane keeping assistance systems (LKAS) — Performance requirements and test procedures,” 2014.